СИНТЕЗ И АНАЛИЗ НА МЕМРИСТОРЕН ЧЕСТОТЕН УМНОЖИТЕЛ

Стоян Кирилов, Георги Ценов, Валери Младенов

Резюме: Основната цел на настоящото изследване е представянето на възможността за използване на мемристора като нелинеен елемент в схемата на честотен умножител. Представен е модифициран мемристорен SIMULINK модел с прозоречна функция на Йоглекер, който е използван във веригата на честотния умножител. Основната идея, използвана при модифицирането на мемристорния нелинеен модел е въвеждането на ненулева минимална стойност на променливата на състоянието, която е свързана с минималния обем на легирания участък на мемристорния елемент. При конструирането на веригата на честония умножител трябва да бъде осигурен режим на твърдо превключване на мемристорния елемент, който е необходим за получаването на богат спектрален състав на мемристорния ток. За отделяне на определен хармоник е използвана проста резонансна верига. Действието на схемата на мемристорния честотен умножител е представено чрез прилагане на симулации в среда на MATLAB и SIMULINK.

Ключови думи: честотен умножител, ограничение, режим на твърдо превключване, спектрален състав.

# Synthesis and analysis of a memristor

# frequency multiplier

Stoyan Kirilov, Georgi Tsenov, Valeri Mladenov

Abstract: The main purpose of the previous investigation is to present the possibility of using the memristor as a nonlinear element in a frequency multiplier. A modified SIMULINK memristor nonlinear ionic drift model with Joglekar window function is presented and used in the frequency multiplying circuit. The basic idea in the modification of the memristor nonlinear model is to introduce a non-zero minimal value of the state variable which is associated with the minimal volume of the doped region of the memristor element. For constructing the frequency multiplying circuit a hard-switching operating mode have to be ensured for the memristor element, which is needed for obtaining a rich spectral structure of the memristor current. For selecting a given harmonic a simple resonant circuit is used. The operation of the memristor frequency multiplier is simulated in MATLAB & SIMULINK environment.

Keywords: frequency multiplier; limitation; memristor; hard-switching mode; spectral structure.