ХАРДУЕРНА СИМУЛАЦИЯ НА СИСТЕМА ЗА АДАПТИВНО

УПРАВЛЕНИЕ С ПРОГРАМИРАНО УСИЛВАНЕ НА

СКОРОСТТА НА ХИДРОАГРЕГАТ

Цоньо Славов, Теофана Пулева

Резюме: В статията е представена разработената система за хардуерна симулация на адаптивен регулатор с програмирано усилване за управление на скоростта на хидроагреагат при наличие на случайно изменящ се в широки граници неизмерим товар. Синтезираният адаптивен регулатор е реализиран в програмируем логически контролер Schneider M258, който управлява в реално време симулатор на процесите в хидрогенератора. Заданието в системата за управление се формира на базата на оценка на товарното смущение, получена от адаптивен наблюдател на състоянието. Управляващият алгоритъм осигурява качество на управление и нулева статична грешка спрямо желания коефициент на статизъм. Представени са резултати от SIMULINK симулация на системата за управление и от симулация с реалното управляващо устройство в контура за управление. Близостта между тях показва работоспособността на реализирания в промишления контролер алгоритъм за управление.

Ключови думи: управление с програмирано усилване, хардуерна симулация, управление с PLC

HARDWARE IN THE LOOP SIMULATION OF SYSTEM FOR GAIN SCHEDULING CONTROL OF HYDROTURBINE SPEED

Tsonyo Slavov, Teofana Puleva

Abstract: In this paper the developed system for hardware in the loop simulation of gain scheduling controller for hydro generator speed control in presence of load disturbance stochastic variations is presented. The control algorithm designed is implemented in Schneider Electric M258 programmable logic controller. It controls the developed real-time hydro generator simulator. The control system reference is calculated based on load disturbance estimate obtained by adaptive state observer. The proposed control algorithm ensures a control system performance. Results obtained by both Simulink simulation and hardware in the loop simulation are presented. The closeness between them shows workability and performance of the developed system.

Keywords: gain scheduling hydro turbine control, HIL simulation, PLC control